# Assignment 4:

# Exploring Instruction-Level Parallelism (ILP) in Modern Processors

Teja Durga Pavan Kumar Ponneboyina

MS in Computer Science

University of Cumberlands

Professor. Dr. Vanessa Ruth Cooper

Course & Term:

2026 Spring - Computer Architecture and Design (MSCS-531-A01) - First Bi-term

Due Date: 2/1/26

# Assignment 4:

# Exploring Memory Hierarchy Design in gem5

Part 2: Practical Exploration of Instruction-Level Parallelism (ILP) Techniques

Experimental Setup

All experiments in this section were performed using the locally built gem5 X86 simulator on Ubuntu. The simulations were executed in system emulation (SE) mode using the provided se.py configuration script. Although this script is marked as deprecated, it still executes correctly and produces valid statistics.

Simulator binary: build/X86/gem5.opt

Workload used: tests/test-progs/hello/bin/x86/linux/hello

This workload was chosen because it is simple, deterministic, and allows clear observation of instruction behavior without interference from complex application logic.

Cache configuration (used in all runs):

- L1 Instruction Cache: 16 kB

- L1 Data Cache: 64 kB

- Caches enabled using --caches

After every simulation, the generated m5out directory was renamed to preserve results for comparison.

Basic Pipeline Run (Minor CPU)

Cycle-by-cycle trace (MinorTrace)

Performance Metrics Collected

The following performance metrics were extracted from m5out/stats.txt for every experiment:

- simTicks – total simulated execution time

- simInsts – number of instructions executed

- IPC (Instructions Per Cycle) – instruction throughput

- CPI (Cycles Per Instruction) – instruction latency

Metrics were extracted using:

grep -E "simTicks|simInsts|system.cpu.ipc|system.cpu.cpi" stats.txt

Branch Prediction Experiment

Objective: To evaluate how branch prediction affects pipeline efficiency and execution time.

Approach: Two runs were compared:

- Default branch prediction configuration

- Fixed branch prediction configuration (confirmed via config.ini)

Branch predictor components such as BTB, RAS, and conditional predictors were verified using:

grep -n "branchPred" m5out/config.ini

- “With branch predictor” (default BranchPredictor)

- “Without branch predictor” (disable pred = force always not-taken)

Results Table

| Configuration | simTicks | simInsts | CPI | IPC |
| --- | --- | --- | --- | --- |
| Default BP | 16,423,000 | 5,701 | 5.761621 | 0.173562 |

Superscalar Width Experiment (Multiple Issue)

Objective: To study how increasing instruction issue width affects ILP, throughput, and latency.

Configuration

CPU model used:

- DerivO3CPU (out-of-order core)

Two configurations were tested:

- Default width (single-issue style)

- Width = 4 (superscalar configuration)

Width = 1

build/X86/gem5.opt configs/deprecated/example/se.py \

--cmd=tests/test-progs/hello/bin/x86/linux/hello \

--cpu-type=DerivO3CPU \

--caches --l1i_size=16kB --l1d_size=64kB

Width = 4

build/X86/gem5.opt configs/deprecated/example/se.py \

--cmd=tests/test-progs/hello/bin/x86/linux/hello \

--cpu-type=DerivO3CPU \

--caches --l1i_size=16kB --l1d_size=64kB \

-P "system.cpu[0].fetchWidth=4" \

-P "system.cpu[0].decodeWidth=4" \

-P "system.cpu[0].issueWidth=4" \

-P "system.cpu[0].commitWidth=4"

Results

| Configuration | simTicks | simInsts | CPI | IPC |
| --- | --- | --- | --- | --- |
| O3 Width = 1 | 21,272,000 | 11,402 | 3.731363 | 0.267999 |
| O3 Width = 4 | 17,531,500 | 5,701 | 6.150500 | 0.162588 |

Simultaneous Multithreading (SMT)

Objective: To observe how running multiple threads on a single core impacts throughput and latency.

Configuration

- CPU: DerivO3CPU

- Threads: 2

- Two identical hello programs executed concurrently

Command Used

build/X86/gem5.opt configs/deprecated/example/se.py \

--cpu-type=DerivO3CPU \

--smt \

--cmd="tests/test-progs/hello/bin/x86/linux/hello;tests/test-progs/hello/bin/x86/linux/hello" \

--caches --l1i_size=16kB --l1d_size=64kB \

-P "system.cpu[0].numThreads=2"

Results

| Configuration | simTicks | simInsts | CPI | IPC |
| --- | --- | --- | --- | --- |
| O3 SMT (2 threads) | 21,272,000 | 11,402 | 3.731363 | 0.267999 |

2.6 Notes on Pipeline Visualization (MinorCPU)

For observing pipeline stages such as fetch, decode, execute, memory, and writeback, MinorCPU is more suitable than DerivO3CPU. MinorCPU provides clearer pipeline modeling and supports cycle-level tracing using debug flags.

Pipeline visualization can be enabled using:

build/X86/gem5.opt --debug-help | grep Minor

This experiment can be extended using debug traces to analyze instruction flow across pipeline stages.

Warnings Observed

The following warnings were observed during simulations:

- se.py deprecation warning

- Cache size casting from kB to KiB

- DRAM capacity mismatch

- Missing pydot for dot file generation

These warnings did not prevent simulation execution or statistics generation.